Using manual of visiting network

This Digital video recorder system can supply two visiting ways for user. One is directly visiting by software of XNETPLAY and IE. There have local area network and wide area network with different configurability of network.

Local network.

Follow the picture, in local network, client computer (192.168.0.2 and 192.168.0.3) need visiting server computer (192.168.0.1), please use inner network IP address: 192.168.0.1.

Internet network

Follow the picture, in internet, client computer need to visiting server computer, please use domain name (e.g. vcdvr.vicp.net)

Former setting for server

Which ways user to use to visiting the network. Must config the former server well first (dvr)

User select “system setting” system reference setting”, please see the picure.

Web station main catalog: means lay web and control full path, please click [browse], select DVR catalog (wwwroot) file (usually C:\VC-404 (8) B-English\server\wwwroot). In default, system may set this directory automatically, User just need to check it.

Web IP address : User only need to use in local network, please fill this computer IP address(dvr IP address) of lan IP, if user need to visit from internet too, please fill the domain name in( e.g. vcdvr.vcip.net).

Router setting

Only setting the router when user wants to visit for internet.

80：HTTP service

5050-5052：receiving images

6789：PTZ control

5040： Remote control , information state、alarm message.

7050： User login check，remote recording playback.

Visiting by client software XNETPLAY2. Please refer to DVR user manual.

Visiting by IE

Open IE, input IP address of former server (dvr) (e.g. local network:192.168.1.61, internet: http://vcdvr.vicp.net:8080)

Remark: Please config the web IP address follow the above specification. Or it should not be connected to. If http port was amended (not 80), please add the port behind domain name (e.g. http://vcdvr.vicp.net:8080). If clients user XP system, please open Active X control of internet setting.

Visiting the former server for first time, system will mention you to download ActiveX control and plug-ins.click [yes], system will install it automatically. Then will pop-up the following picture later.

User ID: 1   password: 1

Remark: The user ID and password are setting for former server (dvr) existing user, user also can add other user to login on former server (dvr).

Follow the picture, after login, user can receive images after double click one of channel.This ActiveX control supply PTZ remote control, remote switch control, remote adjust color, remote playback and so on.