Remote Control Monitoring System

Clinet

User Manual

2nd 2007

Index

1.System introduction	3

1.1 System summarize	3

1.2 System main function	3

2.System setup	3

2.1 environment required	3

2.2 install method	3

3.System operating	4

3.1 system login	4

3.2 interface specification	4

3.3 local set	5

3.3.1 switch set	6

3.3.1.1 append server	7

3.3.1.2 set switch receiver	10

3.3.1.3 use e-map	12

3.3.1.3.1set e-map	12

3.3.1.3.2 use e-map	14

3.3.2 recording setting	14

3.3.3 basic setting	15

3.3.3.1 user management	16

3.3.3.2 set auto login	16

3.3.3.3 default setting	17

3.4 Server list（receiving images）	17

3.5 User logout	18

3.6 PTZ remote controll	19

3.7 Switch remote controll	19

3.8 Adjust color by remote controll	20

3.9 Recording Playback 	21

3.9.1 Local recording Playback	21

3.9.2 Romote recording playback	22

3.10 Image switch	24

3.11 Server alarming	25

3.12 Log management	27

3.13 Right key function meau	27

3.13.1 Right key meau for receiving image	27

3.13.2 Right key meau for server list of right controll panel	28

3.13.3 Right key meau for find recording file	31

3.13.4 Right key meau for switch setting of server	32

3.14 Logout System	33

# 1. System introduction

- 1.1 System summarize

Remote Control system is explored by our company, which are we combine so many years explore experience and take more opinions from our entire customer. This system has new interface, convenient and agile user management, and more strong function, more stable than before.

- 1.2 System main function

IE browse: Browse the monitor image of digital video recorder.

E-map     : support E-map, multiplex display, server list

Clients recording: record compressed data file which sent by net

PTZ control: Control each channel of dvr by PTZ and lens.

Switch control: Control the alarm box output which connect by DVR

Remote config: control the parameter of DVR by internet.

Remote playback: Playback the recording file of DVR by internet

Remote adjust color: Adjust the image of light, contrast, saturation, tinge.

Remote alarm: Receive alarm news of DVR and server, then inform user with audio, video image and so on.

Log management: supply the alarm news and others to user so that he can check it.

# 2. System setup

- 2.1 Environment require

CPU: main frequency must up 1G. (2G needed if receive 16 channel image synchronous)	。

VGA card:  must support hardware zoom play (e.g. GeForce 2 or up)

- 2.2 Install method

Copy the whole file to C:\ (c: \client), then run c:\ client\XNetplay.exe to open the system

# 3. System operation

# 3.1 system login

System login user ID: admin, password: admin. User can add new user or edit existing user by local setting.

“Visit centrol server” in usual, need not to fill, it is only used when the system  in center control system.

“Visit center server” :checkout and warranty the user by center server.

Not use “center server” :only check out and warranty the user by this system. All the user who list in the system can login in.

- 3.2 interface specification

Follow the picture, the interface have 3 parts: center display area, right control panel, lower control panel.

Center display area: display receiving images and recording images playback, support 1.4.9.16.25.36 part screen display. Right control panel: display date and time, supply this system’s main adjust setting function.  Lower control panel: supply the function fast play, stop, grasp and so on

- 3.3 Local setting

Login in the system, user must make local setting before receiving the image of DVR or other device

User can make the “switch setting” “recording setting” “basic setting” like this.

- 3.3.1 Switch setting

It is to preveiw the image by switching mode automatically when the reveiving cameras’ number are more then default display windows(maximal 36).

Switch setting has two dialog boxes for right and left dialog. Left dialog (server list dialog) use to add the server which receiving image. Right dialog (receiving channels dialog) use to set the switch and receiving image.

- 3.3.1.1 Add server

User click [ local setting] ->[ switch setting], Bid the right key on the blank space place, then will pop-up the following program

Please fill the layer actually

User can pitch on “layer”, and then click the right key mouse. It will pop-up the picture (as above), you can set new server on current layer, or reset a new layer.

User click “renew server”, it will display like this picture.

User can “click right menu config server list so that use conveniently.”

Fox example

Config the server list, user can add, delete, edit server by click the right key menu.

- 3.3.1.2 Set switch receive

- Please click the right control panel [local setting]->[switch setting]

Follow the picture, if user need config switch setting, he must be set the server list at first (please refer to 3.3.1.1 add server)

After config the server list, user can select one camera from server list by mouse, or select one channel, then click, system will move server or camera channel to right receiving channel list, pay attention to arrow signal use.

：Move server or camera channel to right receiving channel list

：Move the some channel of current receiving channel list to server list.

：Adjust the sequence of receiving channel list, move ahead.

：Adjust the order of receiving channel list, move backwards.

The sequence of receiving channels is the sequence of switch receiving images, user should edit the sequence of receiving channels with arrowhead, then adjust , then press [ok] to logout setting.

Follow the picture, user select any one server which is right control panel server list. Click right mouse, select “start timing switch” from pop-up menu. Then can receive images automatically at switching mode.

- 3.3.1.3 Use e-map

- 3.3.1.3.1 Set e-map

Please click right control panel [local setting]->[switch setting]->[E-map] key:

Remark: Use E-map, user must set server list well first (please refer to motioned 3.3.1.1 add server).

Follow the picture, select “layer” or “server” of right server list, click [setting E-map] key, system wills pop-up the following picture.

Select images file of map, click [open] key.

Remark: User must prepare the picture (BMP) of map in directory:\client\map.

Follow the picture, user can pulls that layer server to the map moving arclickrarily location. To the server, then, may drag the server, symbled as camera shooting guns, as follows:

After setting the E-map, click [confirm] go back “local setting” -> “switch setting” dialog, please remember to select “Use E-map”

- 3.3.1.3.2 Use E-map

Follow the picture, click centre display "electron map display ", double click server only is capable to do entrance that server map on layer of maps, double click the camera on server E-map, will display camera images.

- 3.3.2 Recording setting

Please click right control panel [local setting]->[ recording setting]

Recording catalog: user can click […..] to edit recording place.

Automation division picture recording file gap: Be how long to generate a recording file.

Automation deleting old recording file: follow the red dialog, mark it, it will delete old recording file when the HDD disk space less than 1G.

Remark: System support manual record temporarily, not support recording by time list.

After setting, user click right key mouse in pane of receiving images. Then choose this pane in the bounding menu, refer to below picture

- 3.3.3 Basic setting

Please click right control penal”local setting” then click basic setting

See the picture, basic setting including User administer setting, Auto signing setting and acquiescence screen-departures setting.

- 3.3.3.1 User Administer

User choose the account which need amend then process operating user account thru click button “adding”, “amending” and “delete”

Remarks:

User Level: Administer have the top authority can make any operation the to the system. Operator can’t set any other function except receiving images, review images

- 3.3.3.2 Auto Login Setting

While Auto login in, that’s to say if system login in auto. We can start to system operation, no need input User name, passport.

Follow the picture, we can only input the user name and passport in the pane (attention, the user should be system existing accountant)

- System start switch auto after login: select it, switch receiving images will be auto.

Remarks:  tick this control means permit auto login or cancel

- 3.3.3.3 Acquiescence setting (disport screen, saving log days)

4 screen showing is the acquiescence system, User can amending screen quantity here

Log saving days: setting the days of log saving, max 90 days

Use the center control flat: select it, then system will validate & authorize user thru center server. Normally, user no need input center server id. Unless the system have add center control system.

- 3.4 Server List (Receiving Images)

Please click right control panel”server list”, it main function is that user can select receiving certain channel images which is from certain server. If found no any other server, it means system have no adding any server information, need adding please refer to “local setting” above mentioned “3.3.1.1 server adding”

Follow the picture, user can receive this server images in the center show area thru double click certain channel

If user needs to stop video images, please select this images pane then right click mouse, then select “stop video” button in the bounding menu. ”stop all” means stop receiving all images.

Remarks: Any other functions you can refer to “right key menu” in next text

- 3.5 User Logout

Please click right control panel”User login”

Follow the picture, press “logout”, then input other users

- 3.6 PTZ remote control

Please click right control panel [ PTZ remote control]

Remark: user must select the picture which needs to control first, then control by PTZ

- 3.7 Remote switch control

Remote switch control, Control the alarm box output of remote control system can control on/off the alarm belling, alarm light, please click right control panel [remote switch control]

。

Remark: user must select the picture before control the picture

Follow the picture, please select the connector which you want to control (support the maximal 24 channels), then press [turn on][ turn off], can control the remote alarm box on/off .

- 3.8 remote adjust color

Please click the right control panel [remote adjust color]

Remark: user must adjust the color by select the picture which you want to adjust.

- ：light

- ：contracts

： tinge

：saturation

When user adjust the color can’t satisfy you, please click [Default] to come back.

- 3.9 Recording playback

The systematic centre display area, not only can receive an image, but also can playback the recording.

- 3.9.1Local recording playback

Local recording playback, only support open the file playback, not support playback by search.

Remark; local playback, user must recording local, specific please refer to 3.3.2 recording setting.

Following the picture, please click [open file], select the file which you want to play from pop-up dialog.

Remark: Recording file was saved “recording catalog” of 3.3.2 recording setting. User want to search the file,. Should press” recording catalog’->recording date->channel->recording file to find and play.

- 3.9.2 Remote control recording playback

Please click right control panel [recording playback]

Follow the picture, user select finding term fist (date, time, type, server and channel ), then press[ search], will pop-up the following picture :

click any recording file can play remotely (remark, remote playback cant play fast), or select any file, click right mouse” backup’ then can backup the recording file on this computer from control system.

- 3.10 image switch

Image switch is the part screen number of center display area, system only support 1.4.9.16.25.36.

Please click right control panel [image switch] to switch.

Follow the picture, when recording images over part screen number, can turn over by press [last] [next]

- 3.11 Server alarming

Server alarming is whether receive alarm information from DVR or the former device. Select server, means alarming server. This system will receive alarm information of this server. If not select server, means quit alarming server, this system will not receive alarm information of this server.

Please click right control panel [server alarming…]

Follow the picture, user select the server which you want to alarming, press[ confirm], the system will pop-up the following picture when receiving alarm information.

- 3.12 log management

Please click right control panel [Event log]

Follow the picture, user can check alarm information and receive server information of image failed.

- 3.13 Right key function menu

After local setting, most of function of system must use right key menu. The follow will sum up the right key function menu of system; let the user command the system more convenient

- 3.13.1 Right key menu of receiving images

“Stop video”: Stop this channel’s (selected channel) images

“Stop all”: Stop all channel images of center display area

“Full screen/natural”: Full screen: full screen display in center display area; please double-click this channel to zoom out the channel

“Start recording”: recording the channel, after select, this menu will change into “stop recording”, click will stop local recording, and need playback local recording please refer to above text “3.9.1 local recording playback”

- 3.13.2 Right key menu for server list of right control panel

Remark: Right key menu “start all’ and “start timing switch” are all the channels of above receiving channel list. User select whichever layer or server, the images are all receiving channel list

Remote config: set parameter on the system setting page.

Follow the picture, specific specification of remote control config please refer to << user menu>>

[Server state]

Follow the picture; server will display recording state and use of hard disk for all the channels. User can on/off control recording for the system all channels by press [turn on recording][turn off recording]

Remark: when DVR have record according to time, it can’t control of recording on/off.

[Remote on/off] : control on/off the system.

Remark: in usual, this system not supply “server state”” remote on/off” function, can supply only this system join control system.

- 3.13.3 Right key menu for find recording file

Press [backup] can download control system recording file to local hard disk.

- 3.13.4 Right key menu for switch setting of server

Follow the picture, user can renew, edit, delete “layer” “server” “channel” through right key menu.

- 3.14 Logout system

Follow the picture, press [yes] can logout the system.