Server Users Manual

2007-02

Catalogue

1．System introduction	4

1.1 System summary	4

1.2 System function	4

1.3 System characteristic	4

2．	System Logging in	5

2.1 Interface Manual	5

2.2 System Logging in	6

2.2.1 Logging out	6

2.2.2 Locking	7

2.2.3 Auto Loading System	7

3．Operation Manual	错误！未定义书签。

3.1 Preview	7

3.2 Recording	8

3.2.1 Manus Recording	8

3.2.2 Schedule Recording	9

3.3 Color	9

3.4 Sensor	10

3.4.1 Manus Switch on Alarm	10

3.4.2 Schedule Switch on Alarm	10

3.5 Bracket	13

3.6 Menu Switch	13

3.7 System Settting	14

3.7.1 System User management	14

3.7.2 System Parameter Setting	14

3.7.3 Picture Attribute & Auto Recording Parameter Setting	错误！未定义书签。

3.7.3.1 Video Attribute	错误！未定义书签。

3.7.3.2 Compression &Transmission Parameter	错误！未定义书签。

3.7.3.3 Auto Recording Setting	错误！未定义书签。

3.7.4 Remove Alarm Video Setting	错误！未定义书签。

3.7.5 Alarm Sensor Setting 	错误！未定义书签。

3.7.6 Bracket Setting	错误！未定义书签。

3.7.7Other Setting (E- Map,Network port,Language management)	21

3.7.7.1 E-Map	22

3.7.7.2 Network Port	22

3.7.7.3 Languange Management	22

3.7.8 Mobile phone Messager	22

3.8 Video Play back 	22

3.8.1 Time Segment Model	23

3.8.2 Document List Model	24

3.8.3 Video files Backup	24

3.9 Picture Snap	25

3.9.1 Picture Snap	25

3.9.2 Picture Preview	25

3.10 Exterior Programme transfer  	25

3.11 Alarm Signal output Control	26

3.12 Advance Place Manegement	26

3.13 Log Manegement	27

3.14 Language Choice	28

3.15 Exit	28

4. FAQ	28

4.1 Server Port	28

4.2 Client Port	29

- System Introduction

- 1.1 System Summary

This System is new generation of Video processor and long time storage which is based on the computer digital picture processing technology. It uses the Philips 7130 chips to carry on the digitization; the real-time compression code after the simulated video signal saves in the system hard disk or through computer communication network transmission realization data sharing. This system is with ability to deal simultaneity 1-24 Channel video signal. Apply to fields such as bank, electric power, telecommunication, water conservancy, intelligence building broadly.

- 1.2 System function

Digital hard disk recording supervisory control system and multi-menu processors

With multi-model of display support 24 channel camera signal input

Under optimum condition, real time display and picture recording may amount to 24 road video frequency

With real time digital video compression

Support simultaneous recording and replay

The pattern, the consumer will easily use the document listing and schedule recording a video replaying

Alarm triggering picture recording

With alarm I|O interface

Give an alarm shifting to an earlier date recording a video in advance

Recording giving an alarm for video displacement

Easy for software to expansion, system integration and update

Support muti-languages

Support the telephone line, male net (ADSL , broadband network) and local area network to carry out each other dyadic online visit and picture recording replaying

Functions such as supporting bracket in advance place, making rounds of visit 	  Displace alarm, the sensor alarm support box output alarm

- 1.3 System Character

■Multi-channel real time: Come true 1-24channel real time compression processing technology. Every passage is independent operation and does not disturb mutually, may adjust on every passage altitude, contrast ratio, chroma, and saturation.

■Longtime recording storage: About 80-120M for one channel one hour.

■Intelligence recording management: A channel can plan recording within one week according to independent schedule, and set the recording time interval within 24 hours. System deletes obsolete recording document voluntarily. Give an alarm according to a channel alarm.

■Inquiry, replay function: The user can seek the needed recording document according to the camera serial number, time section, and event replays together. Replay but free fast-forward, slow way, one by one frame, one by one second, and screen zoom.

■Network transmission function: The network transmission function built-in, Realize Centre centralized recording or Network distributed recording. The network transmission has adopt the standard TCP/IP agreement, making use of "multi-eyes to broadcast” (MultiCast) has decreased by a network occupying bandwidth.

■Effective compression algorithm: System adopt MPEG4/H. 264 algorithms, high compression ratios, and good image mass

■Circumstances changes suddenly checking: System can check the change of light and background in time.

■Characters overlays function: A channel signal can add characters and images on any place.

■System stability: Have set up have "checking extraordinarily" and "systematic exceeding” in hardware and the software a function. System will write the final running information data and restart the computer within 20seconds when meet serious failure.

- System Logging in

- 2.1 Interface manual

As the picture show, the menu is divided as vision area, Right control panel and below control panel.

Vision Area：Display 1、4、8、9、12、16、25 channel images.

Right control panel: Can be set preview sound, picture preview, video, color, sensor, bracket and so on.

Below control panel: Can be do multi-menu switch, system setting, video replay, picture snap, system locking, logging out, exit and so on.

- 2.2 System Logging in

- 2.2.1 Logging out

System Default User Name: super　Key：Null，Click on logging out button [],System will prompt to input user name and key, Following as the picture,

Please enter the user name within 30 seconds, the load on the system, or the system will be locked.

- 2.2.2 Locking

Click on, and then the system will be locked.

- 2.2.3Auto loading system

Click on, Choose ’System Setting’, then click on ’System parameter setting ’label, find and choose the ’Auto Loading system’, Enter the user name and key of auto loading system.

Picture Show, It will load the system in term of the right user name and key.

- Operation Manual

- 3.1 Preview

Click on ‘Preview’ on Right side Control Panel:

As the picture show，Preview panel will reveal the channels by lights , the light green show the channel is previewed and the deep green shows the channel is off.  	[open],[off] Button: It’s easy for user to set system.

- 3.2 Recording

- 3.2.1 Manus Recoding

Click on the Right side Control Panel ’Video Recording’

As the picture show, Preview panel will reveal the channels by lights, the light green show the channel is manus recording and the deep green shows the channel is auto recording; the gray is present recording in term of schedule.

[Open], [off] Button: It’s easy for user to set system.

[

Attention: Schedule recording time, guiding lights becomes a gray, shows being not able to hand movement picture recording.

- 3.2.2 Schedule Recording

Click, Choose ’System setting’, Click on ’Video panel setting ’label, and find ‘Video parameter’

As the picture show, User can add time segment to schedule, then copy to whole week panel or set the detail of every time section in every day.

Attention: From V4.0 now on, Video Recording model the new function can be chosen. Please set time section, daytime use day model, and nighttime use night model.

Video document length: Built video files by time

Attention: Please choose ‘auto loading recording schedule’ when the user has set recording schedule and recording length.

- 3.3Color

Click on Right side Control Panel ’Color’:

As the picture show, adjust channel when be chosen, it can set such as altitude, contrast ratio, chroma, saturation.

[Default]：Come back default settings of color panel

[All the same]：Set for all channel.

[Save]：The settings will be saved to system files.

- 3.4 Sensor

- 3.4.1Manus switch on alarm

Before Manus switch on alarm, Please choose ‘sensor alarm setting’ page, make sure ‘Allow sensor switch on alarm, cancel schedule’ was selected

Click on Right side Control Panel ’Sensor’:

As the picture show,，Sensor panel will show the channel of alarm output box by guiding lights(No guiding lights on shows that disk video recording haven’t linked to output box alarm) the light green show the channel is manus switch on alarm and the deep green shows the channel is manus switch off  alarm, the gray is present recording in term of schedule switch on alarm.

[Open], [off] Button: Set manus switch on alarm or manus switch off on alarm for all channels.

Attention： If selected schedule switch on alarm, guiding lights becomes a gray , shows being not able to manual switch on alarm.

- 3.4.2 Schedule switch on alarm

Click on，Select ’system setting, ’click sensor’ label:

As the picture show, User can set the sensor in term of request.

First, select the type of alarm equipment and the port:

Alarm equipment support: NV808, MV2516, DV7608,W110；please select com1 as communication port; select com2 for other equipment

“Single channel”：Channels of an alarm output box

“All channel”：Total channels, the multi-alarm boxes combines.

For example, there is a 8 channel input and 8 channel output alarm box, please fill in 8,8 .If want to add another one, please fill in 8,16.Follow as this way, it can combine 4 ones more.

Then select sensor alarm

Please make sure the system connecting well with the alarm box, then set the parameters of sensor.

Click sensor 1(first figure label)，select connect video  channels and place for the first sensor, following as the picture:

Attention：please set connect video preview place as ‘3.12preview place management’ Said.

Then set the connect video alarm output parameters：

As the picture show, ‘Associate alarm output channel’, customer can select ‘alarm out’.(V6.0 function)

Attention: The default output need 10 second, output way is close output. Set ‘alarm setting’ in ‘System parameter setting’ label, as the follow picture:

Thirdly，set schedule switch on alarm:

As the graph show, User can add time segment to schedule, then copy to whole week panel or set the detail of time segment in every day.

Video document length: Built video files by time

Attention: Please choose ‘auto loading recording schedule’ when the user has set recording schedule and recording length.

- 3.5 Bracket

As the graph show, user do operations on the bracket, like timing, varifocal, aperture, Iris, Brush, and  move on up or down, left or right and so on.

Please set as’3.7.6 control equipment (camera, bracket) setting said.

- 3.6 Menu switch

Click,System will provide1,,4,8,9,12,16,25 menu switch.

- 3.7 System setting

Attention: Please click on [application] and [confirm] button after finished all parameter settings.

Click on, select ‘system setting’ label, then can set all system parameters setting here.

- 3.7.1 System User Management

As the graph show, System will create two default users ’system management’ and ’Super’, The user can do the operation of revision, increase, deletion according to the actual situation on the system user.

- 3.7.2 System parameter setting

As the graph show, ’System parameter setting’ include Directory path, not enough hard disk disposal way and so no settings

Directory path, not enough Hard Disk Disposal Way: User can keep default setting in general.

If user wants to keep one parts out of recording, rename the parts ‘backup’ (No match case, multi-parts can be renamed ‘backup’).

Attention: User can select the part provided by system, and don’t need to change the path, as the follow graph.

‘Auto Loading System’: Please refer to the ‘2.2.3 auto loading system’:

‘Give an alarm when the video lost’: Select this term, the hard disk will give a buzzer alarm when signal of one channel lost

An alarm happening, allow to giving a buzzer alarm: Selected that the hard disk video recording will give a buzzer alarm if moved alarm or sensor alarm happen.	‘Auto Video Switch’: Select it and adjust the video exchange time, system will exchange the displaying images when it display on the main menu less than actual channel number

Alarm linking video recording time: System will give an alarm when moved alarm or sensor alarm happening according to the time.

Alarm linking output time: Alarm box output the time when the moved alarm or sensor alarm happen.

Alarm trigger alarm image: The image of the channel will change to signal image displaying model when alarm happen.

Output way: The default output is shutting output when alarm happen

Monitor centre having maximal authority for alarm switch on sensor : Select this term, The monitor centre hand the authority of sensor,(For example XNetPlay2).System will response to the sensor triggering alarm signal.

Manus switch on alarm sensor, cancel schedule: Select this term, when User need to manus switch on alarm.

Allowing auto Exchange: Select this term and adjust the time, operation of mouse or keyboard will be not avilable and auto exchange term will be locked.

[Revising system time table]：

Attention: System will turn off automatically and user must restart the system when system time and date is revised, then continue to work. (This function is screened after V 4.0)

- 3.7.3 Image attribute and auto video record setting

As the graph show, Please select video channel when set the video record. Look at the figure Label, it will show the channel number.

- 3.7.3.1Video Attribute

As the graph show, video record attribute can set character display and area screened for preview of channel.

The selected area will be screened when star preview screened function, but the recording file will not to screen.

- 3.7.3.2Compression and Transmission Parameter

There are two models of the image compression bite ratio, permanent allocation bite ratio and the changeable bite ratio.

Max bite, image coder bite keep the max coding value on the model of VBR, the speed of max bite will move but not over the max bite on the model of CBR(V4.0  don’t provide this function)

Key frame interval: The key frame adopt image frame of inner frame compression. The default value is 13.

Image compression setting is compression model and image quality setting.

Compression model: Provide two compression way,H.264 and MPEG4.

H.264：High compression ratio, small video recording file, but more CPU resource need.

MPEG4： A little High compression ratio, but less occupying.

Image quality setting , and Image quality setting.

Excellent, max bite：384，Image quality,3

Good, max bite：256，Image quality,4

Standard, max bite：192，Image quality,5

Work model: Include ’video’ and ‘video+audio’. Please select ‘video+audio’ if the hard disk use by video compression card with audio, or select ‘video’.

Video setting include ‘Better image’ and ‘faster video’.

Better image, if improve image resolution, then reduce video frame ratio

Faster video, If improve video frame ratio, then reduce image resolution ratio.

- 3.7.3.3 Auto Video setting

Please refer3.2.2 Schedule Video Record.

- 3.7.4 Move Alarm Video Setting

Move alarm function, Alarm signal inform the customer when system detect the static image changed in alarm area.

As the graph show, Please set alarm area for every channel. Alarm area divides into 11 X 9 unit, click left button of the mouse  （click ’set all’ button, all unit will set as alarm area）, click left button of the mouse and cancel the unit（click ’cancel all’ button, it will cancel all of the unit of the alarm area）.After all alarm area set, adjust the slide button for parameter setting of alarm image quality and detecting sensitivity.

High speed move checking interval: Value range is  0-12， 2 instead of 0 for high speed checking.

Low speed move checking interval, Use for low speed move but for low speed move checking, value range is 0.

Checking sensitivity, adjust move analytic sensitivity and tune move sense sensitivity on coding dynamic. It determine the sensitivity of DSP, grade 0 is furthers , grade 49 is at latest, we recommend grade 30.

After configuring area, Please set schedule switch on alarm, or not avilable for alarm area settings.

As the graph show, V 6.0 increased alarm output function.

Attention: Alarm box must set correctly, and can communication with monitoring the main procedure.

- 3.7.5 Alarm Sensor Setting

Please refer to 3.4.2 schedule switch on alarm

- 3.7.6PTZ setting

Control equipment includes camera, decoder, speed dome and so on. It control by different port and communication agreement. Bracket setting is setting for port and communication agreement.

User must connect decode with hard disk video recording by 485 conversion and STP.

Controls Parameter:

Control port: It default COM1

Attention: COM1 on the mainboard is close to the power.458 conversion connect the port must be same to the ’control port ’label set, and please check communication port of alarm sensor set if it is COM1 then please change to other port. For example COM2, because of one communication port deal with to one control equipment.

Bracket Set:

System displays the camera list according to the actual camera number. User can choose the camera from the camera list, then click[Edit]button, system pop the follow graph, and select the type of the camera, decode address, control agreement and baud rate, and the name of the channel if default.

As the graph show, the type of the camera can’t fix, It must keep consistent for decode address, control agreement, baud rate and decode.

- 3.7.7 Other set（E-Map, Network port and Language management）

As the graph show, User can set E-map, Network port and language management.

- 3.7.7.1E-Map

If find [E-map edit] button turn gray, Please enter ’system user management’ and modify system administrator, select ’Use E-map’.

Click [E-map edit] button.

As the graph show, up director is toolbar, left side is right side is e-map edit area.

Tree form navigation display the frame of whole e-map. Middle node present map of one area, the final node of camera icon present one camera in the map. User can dblclick the node and enter the node of the map, or click the right button of the mouse, then set add, delete, revise and  switch on alarm on the map.

- 3.7.7.2Network port

Network transfers port, HTTP port, Please don’t revise the window default if need mapping. Other port can’t edit, and it edit automatically based of user.

- 3.7.7.3Language management

Display the loading language, It support Chinese and English. User can add other language.

3.7.8Mobile phone message

Mobile phone message, need to add communication model and built in sim card.

- 3.8 Recording Replay

Please click ，It pop menu of video replay:

As the graph show, recording playback include area replay, right control panel and down control panel.

Area replay: replay 16channel at same time.

Right control panel: Date search set, Channel search set and Exchange of two model replay.

Up control panel: Graphs explain (time segment explain) and play control button (files list model).

- 3.8.1 Time segment model

As the graph show, left vertical coordinate present the selected channel, Superscript between 00 to 24 denote one day; Green across denote auto video recording; Red across denote alarm video recording; Blue across denote manus video recording.

Select date and channel in the right control panel, directly click on the video recording of image, It is finished the video file search. Then please press [play] button.

- 3.8.2 File listing model:

As the graph show, User can specify the detail time segment in different time segment. Select the type of video recording (all, auto, manus, alarm), press[research] button, It display video recording on the list according to the search conditions, dblclik to replay.

- 3.8.3 Back up video recording file

Select the files in video list, click [Back up]button:

As the graph show, select the directory and path, click [OK], the files is back up.

Attention: If user want to remove video replay, please select ’remove server’ ,and如input IP address of the remove hard disk video recording.

- 3.9 Picture graph

- 3.9.1 Picture graph

Select the channel, click on ,system will save the picture to folder of则系the file（picture graph path：D:\NVPhoto）。

- 3.9.2 Picture preview

Click on ，select picture preview

As the picture show, select the channel in left tree manager（picture graph path: D:\NVPphoto）,It will display the picture in the vacancy.

- 3.10 Exterior program transfer

Click on，select ’exterior program transfer

User can [add],[run]exterior program transfer.

- 3.11 Alarm box output

Click on,select ’alarm box output’:

If the alarm output box connects exactly to the system, It display the output port list according to the actual alarm box, As the graph show.

- 3.12 Preset Position Management

Click on，please select Advance Place Management, it will pop as the follow picture:

Select the advance palace number (for example 1), move  the camera to the right place by adjust the bracket button, then click[set] ,and then click[transfer] is ok.

Attention: If user carry out advance management, must control the bracket

Bracket set; refer to ‘3.7.6 bracket set’

- 3.13 Log management

Click, select log management:

As the graph show, user can look over the system log here.

- 3.14 Language choose

Click, select ’language chooses’:

As the graph show, It support English and Chinese.

- 3.15 Exit

Click, System pop prompt message, confirm it, and exit the system.

- 4. FAQ

- 4.1 Server

a. It prompts video recording card initialize failure when run the program.

Driver of video haven’t installed correctly, there is two models, firstly, driver have installed correctly or the computer have restarted; secondly, drive don’t match the software, this problem will generally happen after update software, so It must update the driver when update software.

b. Have preview image, but can’t record.

If some computer can’t install compression engine automatically when first run the system, please install the compression engine by manus.（run mpg4codec.exe in system flooder）.

c. System will become very slow after the monitor system running

Please check the use of CPU, if it is over 90%, then must change to the 2.66G CPU upward.

Attention:16channel upward, please use CELERON　D　2.66G  CPU upward.

d. Monitor system go wrong or exit automatically when use IE or client software用to receive the images.

Please change the 2.66G CPU upward.

Attention:16channel upward, please use CELERON　D　2.66G  CPU upward.

e. Video recording file can play by window media player, but can’t replay by xplayer.

Please install the compression by manus.（run mpg4codec.exe in system flooder）.

f. Flower screen appear on preview image

Please set the color to real color 32 bite

g. No image or gray screen when plug into video signal

For lacking power of computer, please plug 350w power (DC output3.3V and arrive to 27A).

- 4.2 Client receive

a. It can’t find the web page when input the server IP address.

As the graph show, please check the directory and IP address in ‘system parameter’ if not correct server

Web site host directory: Please point to ‘wwwroot’ in the system flooder

Web site IP address：please input the monitor system IP address when the receive如the image in the local network; and input dynamic domain name or wide network IP address if only need to receive image from the wide network

Attention: If choose that wide network receive the image and DVR supervise computer but gateway, please configure the port in the router.

80：HTTP server

5050、5051：Image receive

6789：Bracket control

b. It can’t receive image after Click [connect], if client port software to receive the image.

Please check the set and the network is right firstly.

If it is ok, user can receive the image, please install the compression engine（run mpg4codec.exe in system flooder）.

c. ‘Can’t detect softdog exit’

Please click [set],select ‘hard compression’ in the pop dialogue.

Attention:

Network visit ,please refer to<<manual of network visit.doc>>，

Client port software ,please refer to <<manual of remove monitor system.doc>>。